rk3568b1_base_v1.1 board fits Android 11

The key configuration file

1，device/rockchip/rk356x_box/rk356x_box/rk356x_box.mk

2，device/rockchip/rk356x_box/rk356x_box/BoardConfig.mk

3，device/rockchip/rk356x_box/rk356x_box/system.prop

4，device/rockchip/common/device.mk

5，kernel/arch/arm64/boot/dts/rockchip/rk3568-nvr.dtsi

Adaptation：

Set the dts file：

device/rockchip/rk356x_box/rk356x_box/BoardConfig.mk(File modification)

PRODUCT_UBOOT_CONFIG := rk3568

PRODUCT_KERNEL_DTS := rk3568-nvr-demo-v10

Setting The io_domains:

kernel/arch/arm64/boot/dts/rockchip/rk3568-nvr.dtsi(File modification)

&pmu_io_domains {

status = "okay";

pmuio2-supply = <&vcc_3v3>;

vccio1-supply = <&vcc_3v3>;

vccio2-supply = <&vcc_1v8>;

vccio3-supply = <&vcc_3v3>;

vccio4-supply = <&vcc_1v8>;

vccio5-supply = <&vcc_3v3>;

vccio6-supply = <&vcc_1v8>;

vccio7-supply = <&vcc_3v3>;

};

Note: io_domains should be selected in a pop-up at compile time, which must be consistent with &pmu_io_domains in the file rk3568-nvr.dtsi，

An error is thrown if the value of pmu_io_domains is different from that in the file

Set the root permission to run the terminal

system/core/rootdir/init.rc(File modification)

service console /system/bin/sh

class core

console

disabled

user root

group root log readproc

seclabel u:r:shell:s0

setenv HOSTNAME console

Set the default voice and time zone

device/rockchip/rk356x_box/rk356x_box/system.prop(File append)

persist.sys.locale=zh-CN

persist.sys.timezone=Asia/Shanghai

Compile firmware:

1,Setting environment variables

#source build/envsetup.sh

2,Customized development based on rk356x_box

#lunch rk356x_box-userdebug

3,one-click compilation

#./build.sh -AUCKu

4,Flush firmware

Use the official tool RKDevTool_Release_v2.93.zip to import the firmware package update.img

Tool is located in the directory/RKTools/Windows/AndroidTool/